# Manuscript Notes

## Pitch

To guarantee soft-precise landing of a low SWaP-C quadrotor UAV on an unpredictable moving platform using a monocular camera and IMU:

- When the image-based landing dynamics is highly non-linear, especially near the landing surface

- in the presence of unknown bounded system uncertainties in the landing dynamics, which are most prominent near the landing surface due to ground effect.

- When the target dynamics and the target pose rt, and its derivatives vt, at are unknown.

- without relying on the UAV pose rq and its derivatives vq, aq.

## Abstract

This paper addresses image-based soft-precise landing problem of a low SWaP-C quadrotor UAV on an unpredictable moving vehicle under target visibility constraints. Most prior work on image-based landing suffer from instabilities induced by the highly nonlinear landing dynamics  at touchdown, since the system uncertainties are most prominent near the landing surface due to poor target visibility and ground effects. To address this problem, this paper proposes an adaptive sliding mode control strategy with theoretical stability guarantees that incorporates performance and input constraints to achieve desired transient and steady-state performance in the presence of the unknown bounded system uncertainties.

Detailed Lyapunov stability analysis is used to demonstrate that image parameter errors converge exponentially to small prescribed bounds in steady-state, which is shown to be the key to achieving a guaranteed soft-precise touchdown.

Numerical simulations are conducted to demonstrate the robustness of the proposed control strategy against multiple UAV initial conditions with respect to the target and multiple target trajectories and velocities. Finally, performance comparison studies showcase the superior performance of the proposed control strategy in comparison with the alternative control strategies.

## Introduction

If we have  rq, rt,  vq,  vt, target dynamics, one can easily design a controller to achieve guaranteed soft precise landing on a mobile platform (add citation here).

However, it is difficult to have prior knowledge of the target dynamics or estimate it online. Identify previous works on UAV landing which have achieved precise landing using rq, rt,  vq,  vt without knowing target dynamics (add citation here). This is the case of cooperative target when the target is sharing its both position and velocity with the UAV. Identify how each work is communicating target position and velocity data with the UAV controller to generate the required control inputs. Add a consolidated line on how these approaches are tackling the challenge of unknown target dynamics (again citing each approach).

Then again either target pose rt, or its derivatives vt may not be available or they may be too noisy to be reliable. (Add citation here). Identify previous approaches on UAV landing where either rt or vt is not available and point out how these papers are addressing this challenge, along with unknown target dynamics (again citing each approach).

If suppose both target pose rt, and its derivatives vt are not available or reliable enough to help UAV in landing on the target, this is the case of non-cooperative targets (add citation here). Explain in depth how previous works deal with such challenges (add citation for each approach).

Moving ahead, there are issues with the non-vision sensors like GPS, 2D/3D LiDAR, UWB in computing rq, rt,  vq,  vt which impacts the performance of landing controllers (Add citation here).  Identify how previous work in the literature are achieving landing in the presence of noisy rq,  vq. Add a line on why vision sensors are preferred for low SWaP-C UAVs.

There are two visual-servoing approaches––PBVS and IBVS. What are the issues with PBVS and why IBVS is preferred? (Refer Singhal 2025). State the issues with IBVS like the landing dynamics is highly non-linear, especially near the landing surface where unknown system uncertainties are most prominent. Identify how other papers have addresses these issues in their approaches

Finally add a consolidated paragraph on what is missing in each approach linking it to the open research addressed in our work (refer to the pitch).

Here comes a list of contributions:

- The image-guided soft-precise landing problem on a non-cooperative target is addressed by guaranteeing the stability of the highly uncertain and nonlinear dynamics of the open-loop using the proposed VDF-ASMC strategy executing smooth and continuous control action.

- The control gain dynamically adapts to unknown system uncertainties without requiring any prior knowledge of their upper bounds, ensuring the boundedness of the landing dynamics, particularly near the ground where disturbances and nonlinearities are most pronounced.

- The performance constraints are integrated within the Lyapunov stable control law to guarantee the global exponential convergence of the optic flow error to a small Uniform Ultimate Bound (UUB) around the origin while attenuating the impact of actuator faults and actuator saturation.

- A lateral acceleration clamp, which is triggered by an exponentially decaying target image funnel, explicitly ensures target visibility throughout the landing process.

- Numerical simulations are conducted to demonstrate the robustness of the VDF-ASMC strategy against multiple initial conditions of the UAV with respect to the target and multiple target velocities. Finally, performance comparison studies with four state-of-the-art baselines showcase the superior performance of the proposed strategy in comparison with these alternative control strategies.

The rest of the paper is structured as follows. Add the subsequent lines

## PRELIMINARIES \& PROBLEM FORMULATION

### System Description and Quadrotor Dynamics

In this section, we will talk about the difference reference frame as shown in Fig. {frames_planes.pdf}. We will give Newton-Euler equation for quadrotor dynamics while clearly defining all the mathematical symbols.

### Dynamics of Image Parameters

The image features are projected in the virtual image plane to define scale-independent image parameters such as virtual image position, orientation and velocity.

In this section, we will talk about the following image parameters:

- Virtual image position s

- Virtual image orientation α

- Virtual image velocity h

- Virtual image angular velocity ω

We will explain how we are estimating them using target image feature points and define their kinematics/dynamics equation. Then we will talk about the challenges associated with the system dynamics like:

- The image position kinematics is non-linear

- The image velocity dynamics is non-linear and has singularity condition for acceleration control input.

- Limited FoV and target visibility constraints.

Then we will talk about the following sources of system uncertainties:

- Unmodelled uncertainty: Parametric uncertainty, nonlinear dynamics of batteries, actuator nonlinearities, hardware limitations, such as sensor measurement noise, low camera frame rate and large computational time.

- Exogenous perturbations: wind gusts and the ground effect.

### Problem Statement

The objective of this work is to develop a robust image-based control framework which can guarantee soft-precise landing of a low SWaP-C quadrotor UAV on an unpredictable moving vehicle under target visibility constraints. The unpredictable ground vehicle implies that there is no prior knowledge of the target dynamics so that the target pose qt and its derivatives vt can’t be measured or estimated in the closed-loop system and the low SWaP-C quadrotor requires only a minimal sensing suit that is a monocular camera and an IMU. The image-based landing dynamics is highly nonlinear, especially near the landing surface where the unknown bounded system uncertainties are most prominent.

Specifically, given a desired image position sd, orientation αd and radial velocity hrd, the goal is to synthesize suitable combined thrust-torque command u for the quadrotor so that the virtual image position s, orientation α and radial velocity hr converge to predefined and arbitrarily small neighbourhoods of sd, αd and hrd in the virtual image plane respectively, while ensuring that the target image remains in the sensor’s FoV for all the time. More importantly, it implies that the relative UAV-target velocity reduces to zero as the quadrotor reaches the target position in world frame.

Functional Requirement: For a guaranteed soft-precise landing, the absolute relative UAV-target position and velocity in the world frame should simultaneously converge to origin. Mathematically, it can be represented as:

Let us define:

- Relative UAV-target position as rqt=rq- rt, and

- Relative UAV-target velocity as vqt=vq- vt

Then, for a guaranteed soft-precise landing,  rqt→0 and vqt→0 such that for some unknown touchdown time tf and predefined acceptable thresholds δr and δv, rqttf≤δr and vqttf≤δv.

Challenges:

- Target dynamics is unknown.

- Target motion (vt) is unknown.

- Precise location of target (pt) is unknown.

- Absolute UAV velocity (vq)is unknown.

- Absolute UAV position (pq) is unknown.

Assumptions:

- Vehicle dynamics is known

- Sensor has noise

- There are unknown bounded system uncertainties in vehicle dynamics.